# BCS Level 4 Certificate in Network Systems and Architecture

## Sample paper

## 40 questions

## 1 hour

1 Which two ports does SMB use?

A 	20 and 21.

B 	133 and 443.

C 	139 and 445.

D 	139 and 1096.

2 A customer is unable to access a website. An engineer discovers that their computer can ping a web IP address but not www.google.com. What SHOULD be investigated next?

A 	The computer's ARP table.

B 	The physical network cable.

C 	The wireless SSID

D 	The computer's DNS settings.

3 Which of the following is a benefit of using dynamic routing protocols?

A 	Adapts to changes in the network state automatically.

B 	Adapts to security threats found on the network.

C 	Can reboot routers when the network load is too heavy.

D 	Uses the same pre-defined route for redundancy.

4 A Linux device does not boot. The primary partition has lost the boot flag. Which application would be used to repair this fault?

A 	Disk manager.

B 	F10 BIOS setup.

C 	Device manager.

D 	Fdisk.

5 Which of the following is MOST LIKELY to reduce data loss from a cloud storage failure?

A 	Using a single service provider.

B 	Using multiple data centres.

C 	Encrypting data.

D 	Having good cloud security.

6 Which KEY function of business would employee record management be used in?

A 	Finance.

B 	HR.

C 	Marketing.

D  	Sales.

7 The LEDs on the ports at both ends of a wired link between a server and a switch appear to be correct and the link is working, but its performance is very slow. What COULD be a cause of this?

A 	The switch port is set to reject broadcast packets.

B 	The patch cable is broken and should be replaced.

C 	There is a duplex mismatch between the switch and sever network cards.

D 	The server has not authenticated correctly in the switch's ARP table.

8 What functions does a DNS server perform?

A 	Returns IP address for a given system name.

B 	Returns system name for a given MAC address.

C 	Allows access to resources without knowing the server name.

D 	Provides dynamic TCP/IP addresses to clients.

9 Which of the following is a KEY feature of middleware?

A 	Creates a portable GUI.

B 	Manages system monitoring.

C 	Coordinates processes across systems.

D 	Coordinates disaster recovery.

10 A mail server is not receiving external mail. Which DNS record SHOULD be checked?

A 	PTR.

B 	CNAME.

C 	A.

D 	MX.

11 Excessive latency creates bottlenecks in a network, this can lead to a reduction of effective bandwidth. Which of the following is a sign of an overloaded network device?

A 	Loss of packets.

B 	High throughput.

C 	Low far-end crosstalk.

D 	Low near-end crosstalk.

12 If a server room AC fails, which components within the computer will MOST LIKELY be affected and fail first?

A 	CPU.

B 	Hard drives.

C 	DVD drives.

D 	Graphics card.

13 What type of cloud service would a hosted email service be?

A IAAS.

B PAAS.

C SAAS.

D XAAS.

14 Which of the following can cause CPU failure?

A 	Heatsink failure.

B 	Northside bus failure.

C 	Southside bus failure.

D 	VGA failure.

15 How does a stateful inspection firewall know that a packet belongs to a valid session?

A 	The firewall inspects the packet to see what IP address it has come from and the port that it is communicating through.

B	The firewall inspects the packet to see what connections have been opened from the inside of the network to the Internet.

C 	The firewall inspects the packet to determine which domain it has come from.

D 	The firewall inspects the packet to determine the data link information of the

connection.

16 What is the KEY function of type 1 hypervisor?

A 	Acts as the operating system and shares physical resources to virtual machines.

B 	Uses the installed operating system to share physical resources to virtual machines.

C 	Acts as a virtual machine to share physical resources to the operating system.

D 	Uses the virtual machine's installed operating system to share physical resources.

17 A server that is described as a tower has which of the following characteristics?

A 	A system made of blades each acting as a single server system.

B 	A system that can be stacked on top of each other in a rack.

C 	A system that runs virtual machines in a rack.

D 	A free standing system requiring no support of any kind.

18 Which of the following is a cause and effect of a malware infection?

A 	Weak passwords enabling a DDOS attack.

B 	Patched vulnerabilities on a server allowing installation of ransomware.

C 	Unpatched vulnerabilities on a server allowing installation of a crypto virus that

encrypts all files.

D 	Enabling outbound http traffic on a firewall enabling remote installation of spyware.

19 A Windows update has taken place and the sound has stopped working on a user's device. An engineer has checked the that the latest drivers are installed. After a restart the sound still does not work. What further step SHOULD solve the problem?

A 	Download the latest video drivers and install them as they have audio drivers.

B 	Roll back the update and ignore the constant update attempts.

C 	Change the sound card for a more up-to-date card.

D 	Use a USB headset as this doesn't use the on-board sound card.

20 Which of the following is the strongest wireless encryption method?

A WPA2.

B WPA.

C WEP.

D WAP.